Measuring Ocean Waves Using GPS Buoy

This flyer shows the analysis of wave data measured by a single low-cost GPS buoy. Figure 1 shows the basic definitions for the wave height and wave period in a short record of a wave measurement.

Research Concepts:

The application of the high-pass filtering to GPS data in conjunction with the directional wave spectral theory is core concept (See Figure 5).

GPS Input Data:

GPS Coordinates including:

- Longitude

- Latitude

- Height

Wave Statistical Analysis:

Wave heights, Mean zero-crossing period and other wave parameters are determined directly from high- pass filtered GPS height time-series through zero down- or up cross analysis (See Figures 1 and 5).

Directional Spectral Analysis:

Wave directions are determined by Fast Fourier Transformed (FFT) signals (high- pass filtered GPS height, east and north time-series), which is the energy content at each predefined frequency (See Figures 2 and 3).

Figure 1. Defining zero down-crossing waves of a short record of wave measurements

Figure 2. Power spectrum of the GPS height data (in m2 /Hz)

Figure 3. Directional wave spectra of the GPS buoy data

Deliverables:

- Wave Height

- Wave period

- Wave Direction

Other wave parameters can also be delivered by this method like: Significant wave height, Significant wave period, Peak Period, Mean wave Direction and Spreading. Also one can reconstruct the waveform by GPS buoy.

Figure 4 shows the moored-sea GPS buoy used in Taiwanese coastline.

Figure 5 depicts the filtered GPS height data of the moored-sea GPS buoy in Taiwanese coastline.

Table 1 shows the wave parameters estimated in the Taiwanese coastal area by the moored-sea GPS buoy. Figure 3 shows the directional spectral polar plot of the same GPS buoy.

For more information, please contact Professor Hossein Nahavandchi (hossein.nahavandchi@ntnu.no)

Figure 4. The GPS buoy in Taiwanese coastline

Figure 5. GPS Heights of moored-sea GPS Buoy after filtering

Table 1. Wave parameters estimated from the moored-sea GPS buoy in Taiwanese Coastal region

| Wave Parameters | Result |
| --- | --- |
| Wave Height | 17 cm |
| Wave period | 18.78 sec |
| Wave direction | 226.5 degree |